Cold and Dark to Engine Start

- Select SWITCHES / ENGINE in Aviator dashboard.

- Click on the bottom eye icon to show full overhead panel.

- When using MPD218 panel with Aviator Pads, be sure PAD BANK selector position is B

| ACTION | OVERHEAD PANEL | AVIATOR | MPD218 PAD – BANK B |
| --- | --- | --- | --- |
| BATTERY MASTER – ON |  |  |  |
| STANDBY POWER – ON |  |  |  |
| GROUND POWER – ON |  |  |  |
| APU – START Wait for Blue Light illumination on APU panel, also PAD 4 of MPD218 |  |  |  |
| APU GENERATOR 1 – ON APU GENERATOR 2 – ON |  |  |  |
| GROUND POWER – OFF Don’t press button or pad if was already off! |  |  |  |
| AFT & FWD FUEL PUMPS 1 – ON AFT & FWD FUEL PUMPS 2 – ON |  |  |  |
| APU BLEED - ON |  |  |  |
| HYDRAULIC PUMPS A & B – ON ELECTRICAL PUMPS A & B – ON |  |  |  |
| ENGINE 1 – GRD (GROUND) |  |  |  |
| ENGINE 2 – GRD (GROUND) |  |  |  |
| ENGINE) GENERATOR 1 – ON (ENGINE) GENERATOR 2 – ON APU GENERATOR 1 – OFF APU GENERATOR 2 – OFF |  |  |  |
| APU BLEED - OFF |  |  |  |
| APU – OFF |  |  |  |